Technical Specifications & Cost Estimates – Phase III

TECHNICAL SPECIFICATIONS

NETWORK EQUIPMENT – SERVER SEGMENT - L3 SWITCH

Ministry of Health and Social Service Agency corporate network (MAN, WAN and LAN) infrastructure is currently based on Cisco equipment and technology.  The SIMS project will require the addition of a new switch that must be integrated into the existing network topology and must be compatible with protocols for establishing fault-tolerant default gateways as per RFC 2281, i.e. host standby router protocol.

The following are the minimum requirements which must be met:

| Description | Qty |
| --- | --- |
| Cisco Catalyst 4948, WS-C4948-S | 1 |

BLADE SERVERS

The Ministry of Health currently uses Sun (Oracle) Blade Server infrastructure.  The current inventory is:

- SUN Blade 6000 Chassis (1)

- SUN Blade X6450 Server Modules (2)

Current Server Modules have the following specifications:

- Full Height Blade Chassis

- Processors - Four (4) Intel E7450, 2.40GHz 6-Core, 1066MHz FSB, 12MB L3 cache, 90W Memory - 64 GB (8x8GB) PC2-5300 667 MHz ECC Fully Buffered DDR2 DIMMs

- Memory Slots - 24 DIMM slots

- Storage - One (1) 32GB 2.5" SATA SSD with Special Bracket and One (1) 32 GB CF Card, total 64 GB of storage

- NIC - Two (2) 10/100/1000 Gigabit Ethernet interfaces

- HBA - Two (2) Sun Storagetek PCI-E Enterprise 4Gb FC Host Bus Adapters

- Slots/Ports - Two (2) x8 PCI Express

- One (1) x4 bus

- VGA graphics (DB-15 connector)

- Two (2) USB ports for keyboard, mouse, or storage

Existing servers are used for a virtualized infrastructure using VMware vSphere 4.x.  The Ministry of Health will expand the virtualized infrastructure to accommodate the SIMS project.  All application servers for the SIMS project will be virtualized in this environment.  The infrastructure will also be used to house a database cluster for the SIMS (non-virtualized).  The new servers must be located in the existing blade chassis and must be compatible with it.  Sun (Oracle) has declared X6450 server modules to be no longer orderable and proponents will be responsible to ensure compatibility.  New server modules specifications will exceed those of current modules to meet new demands.

The following are the minimum requirements which must be met:

| Description | Qty |
| --- | --- |
| Chassis - Full Height Blade Processors - Four (4) Processors, minimum frequency 2.40GHz, 6-Core Memory - 128 GB (16x8GB) PC2-5300 667 MHz ECC Fully Buffered DDR2 DIMMS Memory Slots – min 24 DIMM slots Storage - Two (2) 72GB, 15K SAS HDD NIC - Six (6) 10/100/1000 Gigabit Ethernet interfaces Management - One (1) 10/100 Ethernet management port HBA - Two (2) Sun Storagetek PCI-E Enterprise 4Gb FC Host Bus Adapters Two (2) USB ports for keyboard, mouse, or storage OS Compatibility - VMware ESX 4, VMware View 4, Windows Server 2008 Enterprise Edition, 32-bit/ 64-bit | 3 |

STORAGE

The current storage solution in the Ministry of Health is a SUN Storage 6580 array.

To meet new requirements, the Ministry of Health should must add a new drive enclosure and expand the storage capacity.  The drive enclosure should be connected to the existing Sun Storage 6580 controller modules and therefore must be fully compatible.

The following are the minimum requirements which must be met:

| Description | Qty |
| --- | --- |
| Drive enclosure with 16 x 450GB 15K RPM FC-AL Drives for existing Storage System | 1 |

SAN INFRASTRUCTURE

The following equipment specifications are necessary to meet requirements. All equipments must be compatible with the existing SUN Storage 6580 array.

The following are the minimum requirements which must be met:

| SAN SWITCH Description | Qty |
| --- | --- |
| SAN Switch – compatible with existing Sun Storage 6580 Controller Modules Fiber Channel Ports – 24; Minimum Active Ports – 16; Fiber Optic SFP Options – 4 Gbps, 8 Gbps; Recognizes 1,2 and 4 Gb/sec devices; Full 8 Gb/sec performance; Switch Bandwidth – 320 Gbps; Power Supplier – Dual; Frame-based Inter-switch Link Trunking; Integrated Fiber Channel Routing; Virtual Fabrics; FICON; | 2 |

| SFP modules for SAN Switches | Qty |
| --- | --- |
| Product name - 4 GB Short Wave FC SFP 1 Pack Product Type – SFP (mini-GBIC) Interface/Port Details – 1x Fiber Channel Data Transfer Rate – 4 Gbps Gigabit Ethernet Compatible with SAN Switches | 16 |

| FC Patch Cords | Qty |
| --- | --- |
| 5m Multimode Fiber optic Patch Cords Core / Cladding Diameter - 50/125 micron; Connector(s) - 2xLC multi-mode – male Connector(s) (other side) - 2xLC multi-mode – male Compliant Standards – OM3 | 16 |

THIN CLIENTS AND MONITORS

The SIMS project will be utilizing a virtualized desktop infrastructure (VDI) based on VMware technology and therefore use thin-client technology.

The operating system for thin clients should be Microsoft XP Embedded. This is because:

- MoH and SSA’s IT systems are built on the Microsoft technology

- MoH and SSA IT staff have expertise and experience in the Microsoft technology

- Low cost maintenance and support availability

- It is easy and free of charge to Implement Printing and Scanning  system on the Windows Embedded operating system

The following are the minimum requirements which must be met:

| Description | Qty |
| --- | --- |
| Thin Client: Certified for View 4.0 Compatible with: View 3.1, View 3.0, VDM 2.1, VDM 2.0 Resident OS - MS Windows XP Embedded SP2 with support for local applications CPU - 1 GHz processor 2 GB Flash memory 1 GB System memory (64 MB reserved for video) Built in dual monitor support (VGA and DVI-D native) Client license with each thin client Includes one parallel, one serial, two PS/2, and six USB 2.0 ports MIC in and Audio out ports 104 key PC keyboard & mouse Warranty and Services: Three-year hardware warranty | 20 |
| LCD Monitor 19”: 19” Active Matrix TFT Viewable size: 19” (nineteen) Resolution: minimum 1280x1024@60HZ Colors: More than 16 million. Response Rate: not more than 15ms VGA or DVI Input Should include all necessary accessories and wiring as indicated in the user manual Ready to run Monitor should operate on AC mains power 220V, 50/60Hz | 20 |

CLUSTER SOFTWARE

The following equipment specifications are necessary to meet the requirements:

| Clustering Software | Qty |
| --- | --- |
| Software must permit the installation and functioning of Microsoft SQL Server 2008 SP1 (OS – Windows Server R2) in a clustered configuration for high availability or disaster recovery - The following SQL Server assets must be protected: user databases, logins, jobs, alerts, operators, extended stored procedures and other server extensions - Should support: - Single failover cluster instance two-node cluster - Multi-instance cluster: - All nodes with active instances - n+1 (n instances with one spare node) - Multi-site failover cluster instance - Guest failover cluster - Supports centralized control over storage administration preferably via a web-console - Permits nodes to be located in geographically distinct locations - Supports data center-wide visibility into the entire storage environment across all major applications, server, operating system and storage platforms - Automates local and wide area application failures - No single point of failure - Advanced application failover logic - Replication integration with zero data loss - Policy-based response to failures - Supports high availability and disaster recovery testing with minimum or no production impact | License for 2 Servers with 4 x Hex Core Processor |
| Support - 1 hour target response time - Telephone access to support engineers during business hours - Free access to software upgrades, updates and patches - Multiple designated callers to vendor support services - Web-based support forums and access to vendor knowledge bases - For a minimum period of 1 year | License for 2 Servers with 4 x Hex Core Processor |

VIRTUAL DESKTOP INFRASTRUCTURE (VDI)

The Ministry of Health uses a virtualized infrastructure from VMware (vSphere 4.x).  Currently the Ministry has a license for 12 CPUs and virtualization is in use.

The Ministry will be pilot testing, with the aim of deploying throughout all Ministry Structural Units, a virtualized desktop infrastructure (VDI) solution from VMware, i.e. VMware View.

The following are the minimum requirements which must be met:

| Virtual Desktop Infrastructure |  |
| --- | --- |
| vSphere 4 for Desktop, View Manager 4, and vCenter Server Foundation media or ESD and licenses including basic support. | 20 Users Minimum |

COST ESTIMATES

| Description | Qty | Unit Price | Total |
| --- | --- | --- | --- |
| NETWORK EQUIPMENTS |  |  |  |
| Cisco Catalyst 4948 | 1 | 14400 | $14,400.00 |
|  |  |  | $14,400.00 |
| BLADE SERVER MODULES |  |  |  |
| Sun (Oracle) Blade Server Modules X6450 or compatible | 3 | $50,932.20 | $ 152,796.61 |
|  |  |  | $ 152,796.61 |
| STORAGE |  |  |  |
| Drive enclosure with 16 x 450GB 15K RPM FC-AL Drives | 1 | $31,864.41 | $31,864.41 |
|  |  |  | $31,864.41 |
| SAN INFRASTRUCTURE |  |  |  |
| SAN Switch – 24 ports. Active 16 ports | 2 | $12,500.00 | $25,000.00 |
| Transceivers for SAN Switches | 16 | $200.00 | $3,200.00 |
| LC-LC 5m FC Patch Cords | 16 | $80.00 | $1,280.00 |
|  |  |  | $29,480.00 |
| THIN CLIENTS AND MONITORS |  |  |  |
| Thin Clients | 20 | $457.63 | $9,152.60 |
| Monitor 19” | 20 | $110.00 | $2,200.00 |
|  |  |  | $11,352.60 |
| CLUSTER SOFTWARE |  |  |  |
| Clustering Software | 2 | $8,087.00 | $16,174.00 |
| Clustering Software Support | 2 | $1,455.83 | $2,911.66 |
|  |  |  | $19,085.66 |
| VIRTUAL DESKTOP INFASTRUCTURE |  |  |  |
| vSphere 4 for Desktop, View Manager 4, and vCenter Server Foundation Licenses and Media for 20 Users | 2 | $2095.00 | $4,190.00 |
| vSphere 4 for Desktop, View Manager 4, and vCenter Server Foundation Support for 20 Users | 2 | $525.00 | $1.050.00 |
|  |  |  | $ 5,240.00 |
| TOTAL |  |  | $264,219.28 |