PR7836585

Dear All,

Kindly submit quotations for following PR7836585

| No | Unit | Description | Price |
| --- | --- | --- | --- |
| 1 | 450 roll | Microfilm reel: Imagelink HD Microfilm 13 (code: 114NXHU) or equal |  |
| 2 | 36 Bottle | Developer for microfilm processing: Imagelink Archive RTU Dev (art code: AR4NYM7 / 1112490) or equal |  |
| 3 | 24 bottle | Fixer for microfilm processing: Imagelink Archive RTU Fix (art code: AR4NYNA / 1112656) or equal |  |
| 4 | 3 roll | Microfilm Leader / Processing Leader |  |

Microfilm supplies specifications

- Microfilm reel: Imagelink HD Microfilm 13 (code: 114NXHU) or equal

- Film type: Silver Halide (0.135mm thickness)

- Film Base: Polyester (0.130mm thickness)

- 35 mm X 100 feet

- Capable to provide sharp, greatly reduced images of diverse materials in high reduction ratios (40: 1 or more) in Planetary and Rotary cameras.

- suitable for high-speed processing at high temperatures in "deep tank" microfilm processors

- Average Sensitivity Index: 80 ASA (20 DIN)

- Permanent anti-static back layer

- Anti-Halation Undercoating (AHU)

- Life expectancy 500

- Meets all criteria according to ISO 18901:2002 and ISO 18911:2010 imaging materials.

- Developer for microfilm processing: Imagelink Archive RTU Dev (art code: AR4NYM7 / 1112490) or equal

- Ready to use in all microfilm processors. No dilution needed

- Shelf life of the chemicals around 36 months

- 5 Liter/bottle or 3.8 Liter/bottle

- Fixer for microfilm processing: Imagelink Archive RTU Fix (art code: AR4NYNA / 1112656) or equal

- Ready to use in all microfilm processors. No dilution needed

- Shelf life of the chemicals around 36 months

- 5 Liter/bottle or 3.8 liter/bottle

- Microfilm Leader / Processing Leader

- 35 mm X 1,000 feet X 5 mil

- Polyester base

- Non-perforated

- Opaque / hazy white color

- Suitable for microfilm preservation, microfilm archiving, microfilm processing.

·    Your price should be valid for 30 days

·    Please kindly send us the latest  17 Nov 2018 ; 15.00 Jakarta time to   putroa@state.gov

·    PO to be applied Government term of payment is 30 days upon received the item/s and proper invoice

·    Please advise that the goods can be shipped to Lincoln Moving & Storage 8420 s, 98031 KENT,WA 98031 or (Jakarta Warehouse for local vendor)

Please don’t hesitate to send me email for any query related to this order.

Sincerely yours,

Ardi Putro

Purchasing Agent

US Embassy Jakarta

Indonesia