Scope of Work

Tile alignment with inclination for storm water drainage system

At the

US Embassy / Astana, Kazakhstan.

The purpose of this statement (SOW) is to raise the level of pavement tiles at the right side of the Main sliding gate on the US Embassy Compound in Astana with inclination to direct storm water to the drainage pit at the left side of the sliding gate. All the works shall be conducted as per ACI, ASTM/specifications requirements and attached drawing, unless otherwise specified by the Contracting Officer.

Цель работ является поднятие уровня настила каменной плитки с права от ворот главного КПП на территории Посольства США в Астане с учетом наклона для направления ливневых вод в дренажный приемник, находящийся с лева от ворот. Работы должны проводиться в соответствии с рекомендациями американских ассоциаций ACI/ASTM и приложенными чертежами или указаниями контрактного офицера.

Preparation the place “through”. The marking of the surface. Current pavement tiles shall be removed carefully from asphalt, the tiles have to be cleaned from tar for future use (NOTE: the same pavement tiles will be reused after the raising the level), asphalt must be dismantled and removed. Gravel layer should be leveled and tamped. Alignment and compaction of the base. Along the boundary of the gravel with the lawn should be dug a trench 30 cm deep from the top level, a width of 150 mm and a length of 4600 mm for armoring with wire mesh and pouring concrete with inclination to the gutter. The concrete pads shall be cast in place concrete with inclination to the gutter. The concrete pads are equal in dimensions 4500(W)*4600(H)*150(h) mm, below are the concrete requirements:

- Cement Type – Sulphate  Resistant D400

- Design Strength – 25Mpа

- Reinforcement – Double layer steel wire mesh at 100mm spacing each way

Top surface of the pad shall be brush finished and remain cleaned pavement tiles glued with fresh melted tar on surface of the pavement.

Current gutter from tiles needs to be removed and made gutter from armored concrete 450(W)*9000(H) mm, with a recess in the channel no more than 60 mm, with appropriate slope to drainage pit.

Подготовка площадки «корыта». Разметка поверхности. Плитка должна быть аккуратно демонтирована и очищена от налипшей смолы и асфальта для будущего использования, (примечание: существующая плитка будет использоваться после поднятия уровня настила). Слой асфальта под плиткой должен быть убран до существующего слоя гравия. Гравий должен быть выровнен и утрамбован. Вдоль границы гравия с газоном должна быть прорыта траншея глубиной 300 мм от верхней точки уровня, шириной 150 мм и длиной 4600 мм для дальнейшего армирования под заливку бетона. Устройство монолитного бетонного основания выполнить на указанном участке с учетом наклона в сторону желоба. Детализированный чертеж основания прилагается. Бетонное основание размерами 4500(Ш)*4600(Д)*150(В)мм, требования к бетонному основанию:

Для устройства бетонного покрытия использовать:

- Цемент ПЦ – Сульфатостойкий Д400

- Прочность – 25Мпа

- Армирование -  двуслойный каркас ВР сеткой, с ячейкой 100мм.

Верх бетонного покрытия обработать жесткой щеткой. Поверх бетонного покрытия требуется выложить очищенную каменную плитку на свежо расплавленную смолу.

Существующий желоб из плит должен быть демонтирован. На месте плиточного желоба требуется залить армированный желоб с учетом уклона в дренажную шахту. Глубина желоба не более 60 мм.

The Scope of Work is to be performed by a qualified contractor for work required at the US Embassy compound in Astana, Kazakhstan. The contractor’s work will be conducted in August 2018. The client is the Government of the USA through US Embassy in Astana. The contractor shall provide all tools, equipment, materials and manpower required to perform the work specified below and shall provide all the materials meeting the performance specification requirements indicated herein.

Работы должны проводиться лицензированной организацией/подрядчиком для Посольства США, работы должны быть выполнены в августе 2018г. Подрядчик предоставляет все необходимые материалы, оборудование и персонал для выполнения работ.

The contractor shall prepare and submit a work plan schedule which has all operations beginning not later than April 25, 2018. Подрядчик должен предоставить график производства работ не позднее 25 апреля 2018г.

All security requirements required by the Embassy Regional Security Officer shall be followed including requirement for name checks in advance of entry onto the site, access requests and identification of vehicles and materials coming on and off the site. Information for personnel and equipment requiring access to the Embassy compound must be submitted at least 14 days prior to the date of site entry.

Working hours will be allowed between 8:00am to 7:00 pm daily, including weekends and holidays.  Weekend and holiday access to the Embassy compound require 48 hour advance notification.