The U.S. Consulate in Ciudad Juarez, Chihuahua, Mexico is requesting quotes for replacement of wood door and metallic frame per specifications below. Please submit all quotes no later than July 4th 2018 by 4:00 pm (mountain time) to CalvilloMA@state.gov . No quotes will be considered after this date and time. Quote should be valid for 60 days. For ALL vendors it is required that you have a current DUNS number and that you are registered in SAMs. No quotes can be considered without meeting this criterion. Mexican vendors must quote in Pesos unless they  have a U.S. Dollar bank account in Mexico. CAGE and DUNS number on quote are mandatory.

- Supply, labor and installation for the replacement of a wood door and metallic frame with the following specifications:

  - Protect and prepare areas to avoid dust spreading out to adjacent areas.

  - Remove the existing door and frame.

  - Work to open the necessary openning at the wall and reinforcement around frame as needed.

  - Paint all wall around the door, (50 sq mt), Benjamin Moore paint, Eco Spec w/B eggshell 1x base, Color 1094 Barbados Sand.

  - Install carpet as needed. Same as existing.

  - Install new metallic frame, painted same color as the wall, but, following manufacturer recommendations.

  - Finishing touches on all wall and floor areas.

  - Install new door and adjust as necessary with (3 PCS) swing clear hinges ball bearing.

  - Install same lock.

- New hollow metal frame Specs:

  - Metal Products Inc. Size 46” x 84” x 6”, Under Laboratory (UL) plate at frame, Fire Door Frame.

- New Size Wood Door Specs:

  - Marca VT, Size 42” x 84”, cat A, 5508-2-2, ML2057 X LSM trim, 1/8 Bevel 1/8 Bevel, Include kick plate 40” x 34” S.S. gauge 20.

- Perform all finishing touches as necessary.

- Cleaning must be performed at all times to avoid the expansion of dust to the adjacent areas, when work is finished clean the whole area (sweep, vacuum, mop, etc.) as necessary when delivering the finished job. If necessary hire a cleaning company especialized on that kind of cleaning job.

- All works for installation need to be performed during one day of the weekend (Saturday or Sunday) between 8:00 am and 8:00 pm.

In adiition to the completion of the job; all manuals, drawings and warranty documentation must be delivered in order to process payment.

All work is to be performed at the following address.

U.S. Consulate General	
Paseo de la Victoria #3650
Fracc. Partido Senecú
Ciudad Juárez, Chihuahua, Mexico C.P. 32543