EMBAJADA DE LOS ESTADOS UNIDOS DE AMERICA SAN SALVADOR

REF. PR7632126 CSL- APPLIANCES FOR DINING FACILITIES AT CSL COMALAPA

This is a Request for Quotation for a Contractor to provide appliances for dining facilities, as follows:

CONVENTION OVEN SINGLE STACK 38" ELECTRIC							1 (EA)

208v or 220v-240v, 60 Hz; 50 Hz, 1 or 3 phase. Electronic temperature control. Double-pane thermal glass door windows. Interior oven lights. 60-minute continuous ring timer with manual shut-off. 150F (65C) to 550F (288C) temperature range. ¼ HP air-cooled two-speed fan motor. Cool down fan mode. Elements-on and temperature-ready indicator lights. Porcelain enamel oven interior with cove corners. 11 positions removable rack guide with 1 5/8”spacing. 5 heavy-duty, positive stop, chrome plated wire racks. Stainless steel front, top and sides. Black powder coated legs with adjustable bullet feet stainless steel, 60/40 split synchronized doors, with single left mounted handle. Stainless steel door seals, stacking kit for double ovens. Front mounted, hinged control panel for easy service access heavy-duty industrial grade insulation in top, sides, back and doors.

TWO DOOER 48" REACH IN REFRIGERATOR								1 (EA)

Capacity: 34 Cu. Ft. Compressor location: bottom. Depth: 32”. Door type: Stainless steel. Height: 75”. Horsepower: 1/3 HP. Length: 32”. Number of doors: 2. Number of shelves: 6. Opening style: swinging. Refrigerant: 134A. Temperature: 33-38 degrees Fahrenheit. Voltage: 115 V.

UNIVERSAL WELL CAFETERIA STEAM TABLE								1 (EA)

½ Size pan capacity 18. 1/3 Size pan capacity 27. ¼ Size pan capacity 36. 1/6 Size pan capacity 54. Amps 24. Depth 32”. Energy source: Electric. Stainless steel wells and body. Heavy-duty sneeze guard. Full size pan capacity: 9. Height: 36”. Leg type: 6”adjustable. Length: 32”. Phase: 1. Voltage: 208. Weight 495 lbs. wells: 9. Width: 120”.

36" COMMERCIAL ELECTRIC RANGE GRIDDLE								1 (EA)

Base type: standard oven. Griddle included. Height: 36”. Length: 36”. Weight: 660. Width: 30”. Fully insulated with non-sag fiberglass. Porcelain oven interior. 40,000 BTU/hr. equipped with one heavy-duty locking chrome plated rack. Controls: thermostat adjustable from 150 F to 500 F. 100% safety pilot with matchless. Push button ignition. Dimensions: 30”wide x 26 ½”depp x 15”high.

ONE DOOR WHITE SWING GLASS DOOR REACH IN FREEZER						1 (EA)

Access type: door. Amps: 9.6. Capacity: 23 cu. Ft. color: white. Compressor location: bottom mounted. Depth 31 inches. Door style: swing glass. LED lighting, digital thermostat. Height: 81 inches. Hertz: 60. Horsepower 1 HP. Interior depth: 25 inches. Interior height: 55 inches. Interior width: 25 inches. Length 31 inches. Number of doors: 1. Number of shelves: 4. Phase: 1. Refrigerant R-134. Sections: 1. Temperature: -10F – 0F. Voltage: 120. Weight 245 lbs. width: 31 inches.

GLASS DOOER REACH IN REFRIGERATOR								1 (EA)

Stainless steel. 1 full sized swinging glass door. Three internal adjustable heavy-duty polyethylene-coated wires shelves. Temperature: 33-38F. Amps: 9.5. Capacity: 10 Cu Ft. compressor location: bottom. Depth: 23”. Height: 54”. Horsepower: ¼ HP. Length: 23”. Voltage: 120V. Width: 24”.

LOW PROFILE EXHAUST HOOD 72"									1 (EA)

Depth: 48”. Height: 18”. Three filters. Length: 48”. Metal quality: 18 gauge. Width: 72”.

STAINLESS STEELWORK TABLE WITH STAINLESS STEEL UNDER SHELF 60”x30”				1 (EA)

Backsplash height: none. Depth: 30”. Height: 36”. Leg type: stainless steel adjustable ABS bullet feet. Length: 30”. Number of shelves: 1. Option: casters. Tabletop material: stainless steel. Under shelf construction: stainless steel. Weight: 95 lbs. width: 60”. Worktable style: work table.

Shipping Address:

COMUSNAVO FOL EL SALVADOR

ATTN.  CARLOS CORNEJO/VILMA CANJURA

2DA. BRIGADA FUERZA AEREA SALVADOREÑA

CENTRO DE MONITOREO COMALAPA

COMALAPA.  LA PAZ

EL SALVADOR

Alternate shipping destination for non-local vendors:

COMUSNAVSO/C4F

BLDG 3, MAYPORT NAVAL STATION

JACKSONVILLE, FL 32228-0001

904270-5856

MARKED FOR: LS1 WALLER/ VILMA CANJURA

CSL COMALAPA

Please address any questions to MorenoG@state.gov

We will receive your quote no later than Wednesday September 19th, 2018 via e-mail at MorenoG@state.gov.