STATEMENT OF WORK

Background:

The Bureau of International Narcotics and Law Enforcement assist the Government of Ukraine in improving its law enforcement agencies’ operational effectiveness and implementing anti-corruption and criminal justice sector reforms.

Work to be done:

Vendor is required to provide x-ray compact portable inspection system (2 items) with licensed software and warranty; 1 walk-through metal detector with warranty; 4 hand held metal detectors with built-in fast battery and HHDS docking station, warranty included; and 2 packages of plastic screening bins, deliver and install all requested equipment at the SAPO and NABU offices.

VAT Policy:

All offers shall be submitted WITHOUT VAT.

Deadline:

Requested items should be delivered to the SAPO and NABU offices (Kyiv, Ukraine) and installed at the SAPO and NABU offices within 30-60 days from concluding the contract.

- X-RAY INSPECTION SYSTEM WITH REAL-TIME DIAGNOSTICS (CHECKPOINT SECURITY: SMALL BAGGAGE AND PARCEL INSPECTION)

Quantity: 2

SPECIFICATIONS

| GENERAL |  |
| --- | --- |
| Tunnel Size (W × H) | 60.0 cm × 40.0 cm or less but not smaller 53.0 cm × 33.0 cm |
| Dimensions (L × W × H) | 142.0 cm × 84.9 cm × 125.5 cm (length limits to 142.0 cm or less) |
| Mobility | Compact, portable, wheeled |
| Conveyor roller | Includes entry roller table and exit roller table, 46 cm × 46 cm or 50 cm × 50 cm |
| Weight | 389 kg or less |
| Conveyor Speed | 23 cm/s (44ft/min) Forward or Reverse |
| Conveyor Height | 200 mm or less |
| Conveyor Capacity | 165 Load or less |
| Construction | Custom Steel Frame / Panels |
| Color | Grey and/or Black |
| Warranty | Warranty in Ukraine on parts and services for a minimum period of 3 years |
| TECHNICAL |  |
| Sensitivity | 38-44 AWG Typical, 38 AWG Standard |
| Spatial Resolution | - 0.8 mm Horizontal, 1.0-0.8 mm Vertical |
| Steel Penetration | 39-37 mm Typical, 37-35 mm Standard |
| Contrast Sensitivity | 24 Visible Levels, 4096 Gray Level |
| X-RAY GENERATOR |  |
| Voltage | 160 kV Operating at 150kV |
| Tube Current | 0.7-08. mA |
| Cooling | Sealed Di-Electrical Oil Bath with Forced Air |
| Duty Cycle | 100%, No Warm-Up Procedure Required |
| Beam Direction | Diagonal |
| Detector | 1152 Channels or more in an L-Shaped Array |
| COMPUTER |  |
| Display Type | 19 LCD Color Monitor |
| Display Resolution | 1280 × 1024; 24 bit/pixel Color |
| Processor | Intel Core 2Quad, 2.66 Ghz, 1333 MHz |
| Memory | 2 GB RAM at 1333 MHz |
| Storage | 250 GB HDD |
| Video Card | 256 MB Video Card, Dual Digital Display |
| Platform | Windows XP Pro, licensed software |
| Backup | Uninterrupted Power Supply (20 min) |
| ELECTRICAL |  |
| Power Requirement | 110VAC – 230 VAC or more, 50/60HZ |
| ENVIRONMENTAL |  |
| Operating Temperature | 0℃ - 40℃ |
| Storage Temperature | -20℃ - 60℃ |
| Humidity | Up to 95% Non-Condensing |
| HEALTH AND SAFETY |  |
| Compliant with Center for Devices and Radiation Health Standards for Cabinet X-Ray Systems; Typical Radiation leakage is less than 0.1mR/hr |  |
| STANDARD FEATURES |  |
| 6 Color Imaging; 9 Quadrant Zoom; Atomic Z-Number Measurement; Auto Image Archiving (50,000 Images); Baggage Counter; Color and Black/white Imaging; Continuous Scanning; Continuous Zoom 2× to 32×; Density Alert; Edge-Enhancement Imaging; Geometric Image Distortion Correction; High Penetration Function; Heavy Duty Roller Casters; Image Annotation; Image Review (100 Images); Manual Bitmap Archive; Material Discrimination; Multi-Tier Accessibility; Network Ready; Organic/Inorganic Imaging; Picture Perfect; Print Image Capable; Pseudo Color; Real-Time Image Manipulation; Real-Time Self Diagnostics; Reverse Monochrome; Vertical Zoom Planning |  |

- WALK-THROUGH METAL DETECTOR

Quantity: 1

SPECIFICATIONS

| GENERAL |  |
| --- | --- |
| Interior Dimensions (L × W × H) | 63 cm × 82 cm × 205 cm or less |
| Outer Dimensions (L × W × H) | 66 cm × 93 cm × 225 cm or less |
| Weight | 66 kg or less |
| Construction | Scratch Resistant Laminate, Heavy Duty Aluminium or Plastic. No wood. |
| Color | Black or grey |
| Warranty | At least one year warranty in Ukraine |
| STANDARD FEATURES |  |
| 22 Screening Programs; 33 Pinpoint Zones; Analog and Digital Filtering for Inference Supervision; Audio/Visual Alarm; Control Outputs; Digital Signal Processor Based Technology; Directional counter with multiple settings; Entry Pacing Lights; Key Lock Security and Multi-Tier Access Codes; Meets IP 55 IEC Standard; Overhead Control Unit – LCD Display, Alarm Lights, LED Bar Graphs and Control Touchpads |  |
| HEALTH AND SAFETY |  |
| Meets international requirements for electrical safety and compatibility; Meets international regulatory requirements for electromagnetic safety; No adverse effects on pregnancy or medical devices |  |
| ELECTRICAL |  |
| Power Requirement | 100 - 240 VAC: 50/60HZ, 55 Watts No Adjustment Required |
| ENVIRONMENTAL |  |
| Operating Temperature | -20℃ to 70℃ or more |
| Storage Temperature | -40℃ to 70℃ or more |
| Humidity | Up to 95% Non-Condensing |

- PLASTIC SCREENING BINS

Quantity: 2 packages (one package of 9 bins)

SPECIFICATIONS: plastic; gray; dimensions, inches (L × W × H) – 20-1/4 × 15-1/4 × 5

- HAND HELD METAL DETECTOR with built-in fast battery and HHDS docking station

Quantity: 4

SPECIFICATIONS

| GENERAL |  |
| --- | --- |
| Dimensions | length: 16.5’’ (42cm), width: 3.25’’ (8.3cm), weight: 17.6 oz (500g), thickness: 1.625’’ (4.13cm) or less |
| Color | Black or grey |
| Warranty | At least one year warranty in Ukraine |
| TECHNICAL |  |
| Audio Frequency | 2kHz Warble |
| Indicators | Speaker, LED Alert Lights: Green – Power On; Amber – Battery Low; Red: Alarm Condition |
| Controls | Power switch (Audible/Off/Silent), Interference Elimination Momentary Switch |
| Battery | 2 × AA size NiMH researchable environmentally friendly batteries, up to 100 hours of normal operation, built-in fast battery with HHDS docking station, plug for Europe |
| ENVIRONMENTAL |  |
| Operating Temperature | -35° F (-37° C) to 158° F (70° C) |
| Humidity | To 95% noncondensing |